- P1 = 3.2 * 10^9 / 1.5

= 2.13 * 10^9 instructions per second

P2 = 2.0 * 10^9 / 1.0 =

= 2.00 * 10^9 instructions per second

P3 = 4.0 * 10^9 / 2.3

= 1.74 * 10^9 instructions per second

- Number of Cycles:

p1 = 3.2 * 10^9 * 10

= 3.2 * 10^10 cycles

P2 = 2.0 * 10^9 * 10

= 2.0 * 10^10 cycles

P3 = 4.0 * 10^9 * 10

= 4.0 * 10^10 cycles

Number of instructions:

number of cycles / CPI

P1 = 3.2 * 10^10 / 1.5

= 2.13 * 10^10

P2 = 2.0 * 10^10/ 1.0

= 2.00 * 10^10

P3 = 4.0 * 10^10 / 2.3

= 1.74 * 10^10

- Execution time = (number of instructions * CPI) / clock rate

New clock rate = (clock rate * 1.2) / 0.7

= clock rate * 1.71

= 2.0 GHz * 1.71

= 3.42 GHz

- P1 = 2.5GHz

P2 = 3GHz

CPI = x

P1:

= 10^6 * (x * (1 * 30%) + (2 * 20%) + (3 * 30%) + (3 * 20%))

= 10^6x * ((0.3) + (0.4) + (0.9) + (0.6))

= 10^6x * ((0.3) + (0.4) + (0.9) + (0.6)) / 10^6

x = (0.3) + (0.4) + (0.9) + (0.6)

x = 2.2

Total CPI for P1 = 2.2

P2:

= 10^6 * (x * (2 * 30%) + (2 * 20%) + (2 * 30%) + (2 * 20%))

= 10^6x * ((0.6) + (0.4) + (0.6) + (0.4))

= 10^6x * ((0.6) + (0.4) + (0.6) + (0.4)) / 10^6

x = (0.6) + (0.4) + (0.6) + (0.4)

x = 2.0

Total CPI for P2 = 2.0

- Clock cycle = CPI * instruction count

P1:

2.2 * 10^6

P2:

2.0 * 10^6

- CPU time = instruction count * cpi * clock cycle time

CPI = CPU time / instruction count * clock cycle time

Nanosecond = 1 ^ -9

Compiler A:

= 1.2 / (1.0E9 * 1.0E ^ -9)

= 1.2 CPI

Compiler B:

= 1.5 / (1.2E9 * 1.0E ^ -9)

= 1.25 CPI

- 1 / clock cycle time = instruction count * cpi/CPU time

Clock rate(frequency) = instruction count * cpi /CPU time

Fb / FA

= instruction count * cpi / instruction count * cpi

= 1.2E9 * 1.25 / 1.0E9 * 1.2

= 1.25 times faster

- CPU time (a) / CPU time (new)

= instruction count * cpi / instruction count * cpi

= 1.0E9 * 1.2 / 6.0E8 * 1.1

= 1.82 times increase

CPU time (b) / CPU time (new)

= instruction count * cpi / instruction count * cpi

= 1.2E9 * 1.25 / 6.0E8 * 1.1

= 2.27 times increase

- Die area = (pi * r^2) / number of dies

Yield = 1/ (1 + defects per area * die area/2) ^2

Wafer 1:

Die area = (3.14 * 7.5 ^2) / 84

= 176.625 / 84

= 2.10

Yield = 1 / (1 + 0.022 * 2.10/2) ^2

= 1/ 1.04673361

= 0.9553529

Wafer 2:

Die area = (3.14 * 10 ^2) / 100

= 314 / 100

= 3.14

Yield = 1 / (1 + 0.031 * 3.14 /2) ^2

= 1/ 1.099708761

= 0.90933166

Cost per die = cost per wafer / (dies per wafer * yield)

Wafer 1:

= 12 / (84 * 0.9553529)

= 0.14953337

= approximately $0.15

Wafer 2

= 15 / (100 * 0.90933166)

= 0.16495631

= approximately $0.17

Wafer 1:

= ( (3.14 * 7.5^2) / 84 * 1.1) )

= 1.912 cm^2 of die area

= 1 / (1 + (0.022 * 1.15) * 1.912 / 2) ^ 2

= 0.9533 yield

Wafer 2:

= ( (3.14 * 10^2) / (100 * 1.1) )

= 2.855 cm^2 of die area

= 1 / (1 + (0.031 * 1.15) * 2.855 / 2) ^ 2

= 0.9055 yield

- capacitive load = (2* Dynamic Power) / (Frequency * Voltage^2)

capacitive load = (2*48)/((3.2*10^9 )*1.2^2)

= 2.08* 10^-8

- dynamic power = 48 - (10 % *48)

= 48 - 4.8

= 43.2 W

static power = 12 - (10% * 12)

= 12 - 1.2

= 10.8W

total power = static power + dynamic power

= 10.8 + 43.2

= 54W

Power = Voltage * Current

60w = old voltage * current leakage

60W = 1.2V * current leakage

Current leakage = 60W/1.2V

New power = new voltage * current leakage

54 = new voltage * (60W/1.2V)

new voltage = (54 * 1.2) / 60

= 1.08V

Old voltage – new voltage = change in reduction

1.2 – 1.08 = 0.12v

CPI after being reduced: 0.6, 0.6, 2.8, 1.4

Original CPI = ((50*10^6*1)+(100*10^6*1)+(80*10^6 *4)+(16*10^6*2))

/

2 * 10^9

= (50 * 10^6 )+ (100 * 10^6 )+ (320 * 10^6) + (32 * 10^6)

/ 2 * 10^9

= 502 * 10^6

/ 2 * 10^9

= 0.251s

New Cpi = ((50*10^6*0.6) +(100*10^6*0.6)+(80*10^6 *2.8)+(16*10^6*1.4))

/

2 * 10^9

= (30 *10^6) + (60 *10^6) + (224 * 10^6) + (22.4 * 10^6)

/

2 * 10^9

= 0.1682s

Change = New CPI/Original CPI

= 0.1682/0.251

= 0.67s

Change is 1- 0.67

Change is 0.33, 33% faster

new average cpi = 0.8 * 0.251s

= 0.2008

0.2008 = ((50*10^6*1)+(100*10^6*1)+(80*10^6 *4)+(16*10^6*2))

/

2 * 10^9

= (50 * 10^6 * x) + (452 * 10^6)

/

2 *10^9

= 50x + 452 * 10^6

/

2 * 10^9

= (50x +452)

/

2* 10^3

401.60 = (50x +452)

- 50.40 = 50x

-1.008 = x

Cannot Improve the CPI since result is negative

New average cpi = 0.7 *0.251s

= 0.1757

0.1757 = ((50*10^6*1)+(100*10^6*1)+(80*10^6 *x)+(16*10^6*2))

/

2 * 10^9

= (80 * 10^6 * x) + (182 *10^6)

/

2 * 10^9

=80x + 182 * 10^6

/

2 * 10^9

= 80x +182

/

2 * 10^3

351.40 = 80x +182

169.40 = 80x

2.1175 = x

= 4- 2.1175 / 4

= 0.4706, 47.06%

Improve by 47.06% percent