4.3.1: Exercises: R-type:24%. I-type: 28%, LDUR:25%. STUR: 10%, CBZ:11%, B:2%

Data memory = LDUR + STUR

Data Memory = 25% + 10%

Data Memory = 35%

4.3.2:

Instruction Memory = R-type + I-type + LDUR+ STUR + CBZ + B

Instruction Memory = 24% + 28% + 25% + 10% + 11% + 2%

Instruction Memory = 100%

4.3.3:

Sign extend = I-type + LDUR + STUR + CBZ + B

Sign extend = 28% + 25% + 10% + 11% + 2%

Sign extend = 76%

4.4.1:

LDUR

4.4.2:

LDUR, STUR

4.4.3:

STUR, CBZ

4.19:

ADDI X1, X2, #5

Instruction adds value of register X2 and value 5, stores in register X!

X1 = X2 + 5

X1 = 22 +5

X1 = 27

ADD X3, X1, X2

Instruction adds value of register X1 and X2 and stores in register X3

X3 = X1 + X2

X3 = 27 + 22

X3 = 49

ADDI X4, X1, #15

Instruction adds value of register X1 and value 15, stores in register X4

X4 = X1 + 15

X4 = 27 + 15

X4 = 42

ADD X6, X1, X1

X5 = X1 + X1

X5 = 27 + 27

X5 = 54

4.30.1:

Instruction 1: CBZ, X1, LABEL

Can cause an invalid target address exception, detected during the Execute(EX)  pipelining stage.

Instruction 2: LDUR X1, [X2, #0]

Can cause an invalid data address exception, detected during the Memory Stage (MEM)

4.25 A.1:

“N” = ADDED FOR ADD INSTRUCTION

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LDUR X10 [X1,#0] | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |
| LDUR X11 [X1,#8] |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |
| ADD X12 X10 X11 |  |  | N | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |
| SUBI X1 X1 #16 |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |
| CBNZ X12 LOOP |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |
| LDUR X10 [X1, #0] |  |  |  |  |  |  | ID | ED | EX | MEM | WB |  |  |  |  |  |
| LDUR X10 [X1, #8] |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |
| ADD X12 X10 X11 |  |  |  |  |  |  |  |  | N | IF | ID | EX | MEM | WB |  |  |
| SUBI X1 X1 #16 |  |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |
| CBNZ X12 LOOP |  |  |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |

4.24 A.2:

“N’ = bubble

“S” = stall

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LDUR X10 [X1,#0] | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LDUR X11 [X1,#8] |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |  |  |
| ADD X12 X10 X11 |  |  | N | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |  |
| SUBI X1 X1 #16 |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |  |  |
| CBNZ X12 LOOP |  |  |  |  |  | S | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |
| LDUR X10 [X1, #0] |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |  |
| LDUR X10 [X1, #8] |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |  |  |  |
| ADD X12 X10 X11 |  |  |  |  |  |  |  |  |  | N | IF | ID | EX | MEM | WB |  |  |  |
| SUBI X1 X1 #16 |  |  |  |  |  |  |  |  |  |  |  | IF | ID | EX | MEM | WB |  |  |
| CBNZ X12 LOOP |  |  |  |  |  |  |  |  |  |  |  |  | S | IF | ID | EX | MEM | WB |