- 16 bytes * 8 bits = 128 bits

128/ 64 bit integers =

2 8 byte blocks

- i, j because they are called in all iterations

- a[j][i], c[i][j] will exhibit special locality.

| Address | Binary | Tag | Index | Hit/Miss |
| --- | --- | --- | --- | --- |
| 0x43 | 0100 0011 | 0100 | 0011 | miss |
| 0xc4 | 1100 0100 | 1100 | 0100 | miss |
| 0x2b | 0010 1011 | 0010 | 1011 | miss |
| 0x42 | 0100 0010 | 0100 | 0010 | miss |
| 0xc5 | 1100 0101 | 1100 | 0101 | miss |
| 0x28 | 0010 1000 | 0010 | 1000 | miss |
| 0xbe | 1011 1110 | 1011 | 1110 | miss |
| 0x05 | 0000 0101 | 0000 | 0101 | miss |
| 0x92 | 1001 0010 | 1001 | 0010 | miss |
| 0x2a | 0010 1010 | 0010 | 1010 | miss |
| 0xba | 1011 1010 | 1011 | 1010 | miss |
| 0xbd | 1011 1101 | 1011 | 1101 | miss |

| Address | Binary | Tag | Index | Offset | Hit/Miss |
| --- | --- | --- | --- | --- | --- |
| 0x43 | 0100 0011 | 0100 | 001 | 1 | miss |
| 0xc4 | 1100 0100 | 1100 | 010 | 0 | miss |
| 0x2b | 0010 1011 | 0010 | 101 | 1 | miss |
| 0x42 | 0100 0010 | 0100 | 001 | 0 | hit |
| 0xc5 | 1100 0101 | 1100 | 010 | 1 | hit |
| 0x28 | 0010 1000 | 0010 | 100 | 0 | miss |
| 0xbe | 1011 1110 | 1011 | 111 | 0 | miss |
| 0x05 | 0000 0101 | 0000 | 010 | 1 | miss |
| 0x92 | 1001 0010 | 1001 | 001 | 0 | miss |
| 0x2a | 0010 1010 | 0010 | 101 | 0 | hit |
| 0xba | 1011 1010 | 1011 | 101 | 0 | miss |
| 0xbd | 1011 1101 | 1011 | 110 | 1 | miss |

| Address | Binary | Tag | Index | Offset | Hit/Miss |
| --- | --- | --- | --- | --- | --- |
| 0x43 | 0100 0011 | 0100 | 00 | 11 | miss |
| 0xc4 | 1100 0100 | 1100 | 01 | 00 | miss |
| 0x2b | 0010 1011 | 0010 | 10 | 11 | miss |
| 0x42 | 0100 0010 | 0100 | 00 | 10 | hit |
| 0xc5 | 1100 0101 | 1100 | 01 | 01 | hit |
| 0x28 | 0010 1000 | 0010 | 10 | 00 | hit |
| 0xbe | 1011 1110 | 1011 | 11 | 10 | Miss |
| 0x05 | 0000 0101 | 0000 | 01 | 01 | Miss |
| 0x92 | 1001 0010 | 1001 | 00 | 10 | Miss |
| 0x2a | 0010 1010 | 0010 | 10 | 10 | Hit |
| 0xba | 1011 1010 | 1011 | 10 | 10 | Miss |
| 0xbd | 1011 1101 | 1011 | 11 | 01 | hit |

C3 is better than c1 and c2 because it has more hits than misses.

| Block size | Miss Rate % | Latency | Access time |
| --- | --- | --- | --- |
| 8 | 5 | 30 * 8 = 240 | 1 + 5% * 240 = 13 |
| 16 | 3 | 30 * 16 = 480 | 1 + 3% * 489 = 15.4 |
| 32 | 2 | 30 * 32 = 960 | 1 + 2% * 960 = 20.2 |
| 664 | 1.5 | 30 * 64 = 1920 | 1 + 1.5% * 1920 = 29.8 |
| 128 | 1.1 | 30 * 128 = 3840 | 1 + 1.1% * 3840 = 43.24 |

Block size of 8 bytes has the fastest access time, it is the optimal block size.

Optimal block size is 128 blocks because it has the lowest miss rate (1.1%).

- Size of word = 64 bit/8

= 8 byte

Size of index = 9 bits

Size of offset = 3 bits

Size of tag = 64 – 12

= 52 bits

- Size of index = 64 blocks

= 2^6

= 6 bits

Size of offset = 8*8 byte

= 2^6

= 6 bits

Size of tag = 64-12

= 52 bits

- In a = 3:64

In b = 6:64

- Size of block = 1 word

= 8 bytes

Number of blocks = size of cache / size of word

= 512/1

= 512

Number of index = 512/2

= 256

= 2^8

Size of index = 8 bits

Size of offset = 3 bits

Size of tag = 64 – 11

= 53 bits