CS 550 Homework Assignment 5

Collaboration Policy. Homework will be done individually: each student must hand in their own answers. It is acceptable for students to collaborate in understanding the material but not in solving the problems or programming. Use of the Internet is allowed, but should not include searching for existing solutions.

Under absolutely no circumstances code can be exchanged between students. If some code was shown in class, it can be used, but it must be obtained from Canvas, the instructor or the TA.

Assignments from previous offerings of the course must not be re-used. Violations will be penalized.

Late Policy. No late submissions will be allowed without consent from the instructor. If urgent or unusual circumstances prohibit you from submitting a homework assignment in time, please e-mail me.

Deliverable: A single file on Canvas. All answer must be typed.

Problem 1: (25 points) N-cube networks are known for their ability to sustain broken links and still provide connectivity between the switches.

- How many links in 1-cube can fail and we still have full connectivity?

- How many links in 2-cube can fail and we still have full connectivity?

- How many links in 3-cube can fail and we still have full connectivity?

- How many links in N-cube can fail and we still have full connectivity? Give your answer in terms of N.

Problem 2: (25 points). Consider the following code, which multiplies two vectors that contain single precision floating point values:

for (i=0; I <100; i++) {

c[i] = a[i] * b[i]

}

We define the Arithmetic Intensity of a code as the number of operations to run the program (for example in C) divided by the number of bytes accessed in the main memory. Arithmetic Intensity is a good measure of arithmetic operations versus the number of bytes transferred between memory and CPU so we can see whether the system is computation bound or memory bandwidth bound as we discussed in class. What is the arithmetic intensity of the above code?

Problem 3 (25 points). Assume a GPU that runs at 1.5 GHz with 16 SIMD processors, each having 16 single –precision FP units. This GPU is supported by a100GB/s off-chip memory. Assume all memory latencies can be hidden. Assume each operation is an addition.

- (15 Points) Ignoring memory bandwidth, what is the peak SP FP operation in GFLOP/sec.?

- (10 Points) Is this throughput sustainable given the bandwidth? Justify your answer.

Problem 4 (25 points). Assume a GPU architecture that contains 16 SIMD processors and has a clock speed of 4 GHz. Each SIMD can produce 24 single precision arithmetic result on its 8 lanes every cycle.

Now Assume we run a program on this GPU that on average 80% of threads are active (due to branches). Compute the throughput, in GFLOP/sec, for this code on this GPU.